(E6.11) A system with a transfer function Y(s)/R(s) is Ts=YsRs=24(s+1)s4+6s3+2s2+s+3

Determine the steady-state error to a unit step input. Is the system stable?

Ans.

Step input Rs=As, Ys=24(s+1)s4+6s3+2s2+s+3As

- eSS=sRs-Ys =A1-Ts =-7A

Poles: 0, -5.67, -0.9, 0.28±0.714j

- The system is unstable due to right-half s-plane poles.

(E6.19) Determine whether the systems with the following characteristic equations are stable or unstable:

- s3+4s2+6s+100=0

- s4+6s3+10s2+17s+6=0

- s2+6s+3=0

Ans.

- Poles: 0.94±4.02j, -5.88 🡺 Unstable

- Poles: -0.44, -4.56, -0.5±1.66j 🡺 Stable

- Poles: -0.55, -5.5 🡺 Stable

(E6.23) A system is represented by Equation (6.22) x=Ax where

A=0 1 0 0 0 1 -8 -k -4 , characteristic polynomial is s3+4s2+ks+8=0.

Find the range of k where the system is stable.

Ans.

characteristic polynomial is s3+4s2+ks+8=0.

The Routh array is

- -141 k 4 8 =4k-84=k-2, -1k-24 8 k-2 0 =8

- For Stability, 4k-84>0⇒k>2

(E6.25) A closed-loop feedback system is shown in Figure E6.25. For what range of values of the parameters K and p is the system stable?

Ans.

Ts=sK+1s2s+p+(sK+1) 🡪 the characteristic equation is s3+ps2+Ks+1=0

The Routh array is

- -1p1 K p 1 =pK-1p, -ppK-1p 1 pK-1p 0 =1

- For Stability, pK-1p>0 & p>0⇒p>0 & K>1p

***** Important Equation in Chapter 6 *****

- Routh-Hurwitz Stability Criterion

  - All the coefficients are nonzero.

  - All the coefficients have the same sign.

- Four Distinct Cases for Routh Array Calculation Procedure

  - Routh–Hurwitz criterion states that the number of roots with positive real parts is equal to the number of changes in sign of the first column of the   Routh array.

  - Four cases: (1) No element in the first column is zero; (2) there is a zero in  the first column, but some other elements of the row containing the zero inthe first column are nonzero; (3) there is a zero in the first column, and the other elements of the row containing the zero are also zero; and (4) as in   the third case, but with repeated roots on the imaginary axis