  - stage1: H1s=-(2gm)(R/2)1+s(R/2)(C)=-gmR1+s/ω10, ω10=2RC

stage2: H2s=-(gm)(R)1+s(R)(2C)=-gmR1+s/ω20, ω20=12RC

  - Hs=-gmR1+sRC2-gmR1+s2RC-gmR1+sRC2-gmR1+s2RC=-gmR411+sRC2211+s2RC2

∠HωOSCopen=-180°×5-2atanωOSCRC2-2atanωOSC2RC=0°

⇒atanωOSCRC2+atanωOSC2RC=90°⇒ωOSC=1RC

HωOSC=gmR41+22-21+0.52-2=425gmR4=1

⇒gmR=1.58

  - Oscillation frequency (ωOSC) is 1/RC.

Additional phase shift of stage1 is atanωOSCRC2=26.56°

Additional phase shift of stage2 is atanωOSC2RC=63.44°

  - VOUT,A=A0→VOUT,B=A0H2ωOSC=A0gmR1+0.52=0.89A0gmR

  - We know that ISS=500μA=2Id2 and Vout,CM=1V, then we can get R=Vdd-Vout,CMId2=3200 Ω

Bandwidth = 1GHz, RC=12πf=12π(1G)=7.96×10-11⇒C=50.8fF

- Specification

| Element | Value |
| --- | --- |
| Stage 1 (M11, M12, M31, M32) | (60μm/0.18μm) |
| Stage 2 (M21, M22, M41, M42) | (30μm/0.18μm) |
| R | 3200 Ω |
| C | 50.8 fF |
| ISS | 500 μA |
| Gain of each stage | 7.99 (18.1dB) |
| Input Common-mode | 1 V |
| Vout | 1 V |

- Stage 1

- Stage 2

- The gain I designed is 7.99, which is close to 5 times value of minimum required low-frequency gain.

- For Stage 1, we expect that the -3dB gain would locate at 2GHz, and it shows up at 1.13GHz in the simulation. For Stage 2, we expect that the -3dB gain would locate at 500MHz, and it shows up at 525MHz in the simulation.

  - This is because we ignore the parasitic capacitance and channel-length modulation in calculation.

  - Use “.option DELMAX” to set the time step to less than 1 ps.

  - Simulation

- All 8 oscillation waveforms

- Stage 1

  - Input: vout8, Output: vout1

  - Amplitude of stage 1 (|vout1|): 580 mV

  - Period of each signal = (5.44n – 4.14n) = 1.29n

total phase shift = ΔtT×360°=4.14n-3.39n1.29n×360°=209.30°

  - Additional phase shift = 180° - total phase shift in one period

Then, we can get the additional phase shift is 29.30°.

- Stage 2

  - Input: vout1, Output: vout3

  - Amplitude of stage 2 (|vout3|): 270 mV

  - Period of each signal = 1.29n

total phase shift = ΔtT×360°=5.02n-4.14n1.29n×360°=245.58°

  - Additional phase shift = 180° - total phase shift in one period

Then, we can get the additional phase shift is 65.58°.

  - Comment

    - Hand-calculation

Additional phase shift of stage1 is 26.56°

Additional phase shift of stage2 is 63.44°

    - Simulation

Additional phase shift of stage1 is 29.30°

Additional phase shift of stage2 is 65.58°

    - Comment

      - The simulated values of -3dB gain both in stage 1 and stage 2 are lower than the expected values. This is because we ignore the parasitic capacitance and channel-length modulation in calculation.

      - Due to ignoring the parasitic capacitance and channel-length modulation in calculation, we got the larger values of additional phase shift in each stage in the simulation.

    - Specification

| Element | Value |
| --- | --- |
| Stage NMOS (M11n, M12n) | (50μm/0.18μm) |
| Stage PMOS (M11p, M12p) | (3.4μm/0.18μm) |
| C | 200 fF |
| ISS | 500 μA |
| Gain of each stage | 8.15 (18.5dB) |
| Frequency of -3dB Gain | 300 MHz |
| Input Common-mode | 1.4 V |
| Output Common-mode | 1.398 V |

    - Waveview of each stage

    - Pole-Zero

    - Prove “the tail current steers fully when oscillating”

      - This result shows that the maximum current flows into M11n and M12n (NMOS) are 503 μA and 492 μA, and the minimum current flows into M11n and M12n (NMOS) are 7.58 μA and 8.22 μA. This also shows that the tail current steers fully when oscillating.

  - VCO frequency vs. Vctrl

    - VCO frequency vs. Vctrl

      - We can estimate the VCO gain at frequency 500 MHz is 0.37 V.

    - AC response vs. Vctrl

  - Simulation

    - All 8 oscillation waveforms

      - Vctrl=0.37 V

      - Operating frequency = 1/(9.18n-7.2n) = 505 MHz

    - Stage 4 (input) and Stage 1 (output)

  - Input: vout8, Output: vout1

  - Amplitude: 22 mV (1.75mV – 1.53mV)

  - Period of each signal = 505 MHz

total phase shift = Δt×f×360°=219.98°

  - Additional phase shift = 180° - total phase shift in one period

Then, we can get the additional phase shift is 39.98°.

    - Stage 1 (input) and Stage 2 (output)

      - Input: vout1, Output: vout3

      - Amplitude: 255 mV

      - Frequency of each signal = 505 MHz

total phase shift = Δt×f×360°=236.34°

      - Additional phase shift = 180° - total phase shift in one period

Then, we can get the additional phase shift is 56.34°.

    - Comment

      - Changing the value of Wn doesn’t affect the value of output common mode.

      - This design doesn’t work when control voltage (Vctrl) larger than 0.8 V.